Firstly, please send me your camera system’s version information, as below picture shown,

Or according to the link below, you can upgrade the camera system by yourself. Please upgrade the camera system to the latest version.

https://anran-cctv.com/faq-items/q-how-to-upgrade-the-wireless-camera-system/

Setting email alarm:

The Cloud ID should be ONLINE and the Network Status must say "Healthy Network" first. Please prepare a Gmail address and set up the email notification as following steps.

Step1. Login to your Google account on your PC, click “Signing in to Google.” Make sure “2-Step Verification” is off and the “Allow less secure apps” is turned on.

Step2. Go to NVR system, right click: System setup→Channel Setup→Video detection. Check Enable for Motion Detection→Open Alarm-mail Notice

Step3. Setup email account

Click right→System Setting→Network Setting→Email. Enable E-mail function and fill in email settings. Click Test until it is successful. If the test failed, please change to another e-mail and try again. Finally, click OK to save all the settings (No matter test success or not, please click “Apply” to save your settings first).

Video instructions:

https://drive.google.com/open?id=1AFaZ2J3F3C1Rj2VsA5ot_4Ru6x1RrL-R

The first setting:

As below picture shown,

Port: 25

Encryption Type: TLS

| SMTP Server | You email smtp server. For example: smtp.gmail.com |
| --- | --- |
| Port | 25 |
| User Name | Sender e-mail address(Gmail e-mail address) to send alarm |
| Password | Sender E-mail login password |
| Encryption Type | TLS |
| Sender | Sender e-mail address to send alarm |
| Sendee1/Sendee2 | Receiver e-mail address to receive alarm |

The second setting :

As below picture shown,

Port: 587

Encryption Type: TLS

| SMTP Server | You email smtp server. For example: smtp.gmail.com |
| --- | --- |
| Port | 587 |
| User Name | Sender e-mail address(Gmail e-mail address) to send alarm |
| Password | Sender E-mail login password |
| Encryption Type | TLS |
| Sender | Sender e-mail address to send alarm |
| Sendee1/Sendee2 | Receiver e-mail address to receive alarm |

The third setting :

As below picture shown,

Port: 465

Encryption Type: SSL

| SMTP Server | You email smtp server. For example: smtp.gmail.com |
| --- | --- |
| Port | 465 |
| User Name | Sender e-mail address(Gmail e-mail address) to send alarm |
| Password | Sender E-mail login password |
| Encryption Type | SSL |
| Sender | Sender e-mail address to send alarm |
| Sendee1/Sendee2 | Receiver e-mail address to receive alarm |